# Water Heater Ntc Thermistor Temperature Sensor Probe

MFP series temperature sensor using the NTC resistance element, according to the different temperature environment or application, through the mature technology, fabricate into a variety of specifications of the sensor, customers can use directly without fabricating.

Technical data

| Item name | Performance |
| --- | --- |
| Sensing element | NTC Thermistor various R and B value on request |
| Temperature range | -30℃~ +150℃ |
| Response time | Water (0.4m/s) T0.63≤30s |
| Dissipation factor | ≥2.5mW/℃ |
| Long-term stability | Drift≤3% after 1000h heat or cold store (80℃/-30℃ |
| Dielectric strength | 1500VAC |
| Insulation resistance | ≥100MΩ 500VDC |

Other options:

- Standard value:R25℃: 10K  B(25/85℃)=3435K, different NTC R-T performance curve available

- Different probe and wire length and color available

- Different size SUS housings available for sensor mounting protection

- Different type of connector is available

Typical applications:

- Solar water heater

- Toilet seat heating

- Floor heating and cogeneration systems

Features

Moisture-proof, fast response and small time constant etc.

Relevant product photo